TEMPEST Autochamber Flux Data Processing Protocol:

- Locate data for the date you are looking for on the google drive: https://drive.google.com/drive/folders/1ETfZoritHHmPS0X4vXFLmjBlqMCS8OK8?usp=drive_link

- Pull STEEP-CF-autochambers GitHub (https://github.com/COMPASS-DOE/STEEP-CF-autochambers/tree/main) to your desktop and create a branch based on the current date (eg., “autochamber-fluxes-20260928”)

- Open the GitHub Desktop in explorer

- Under “Processing Scripts,” create a folder for the day you are processing

- Copy over the Rmd files from the “Templates” folder and rename for the date you are processing

- Open the 8100 processing Rmd and change the Date in the first chunk to the date you are processing as YYYY-MM-DD

- Change the google drive link for the target_folder in the second chunk to the folder in google drive you will be pulling from.

- Run the 8100 code to create a processed 8100 metadata file.

- Check that the 8100 file was created with Plot, obs_length, start, and end times in the Processed_Data folder.

- Open the 7810_CO2 file and change the Date_dashes to the date you are processing in the format YYYY-MM-DD.

- Run the code to pull in 7810 data, create edited metadata, and match metadata to flux data.

- For each plot, the chunk will also graph the first hour of CO2 fluxes, with the collar that should be first in red and the collar that should be last in blue. Check to see that the fluxes were identified in the correct order.

- Run chunks to calculate CO2 fluxes, and graph fluxes with high R squared, low R squared, and high/low flux values.

- Look over the flux check graphs to check for late closing/early opening/ebullition during fluxes.

- If there is an issue with the flux at the start, add to Time_8100 to increase the start time in the “Make changes to metadata” chunk for the Time_7810.

- If there is an issue with the flux at the end, subtract from total_seconds to reduce the duration of the flux.

- If there is a major issue with the flux in the middle, you may need to flag the flux in the metadata as “Flux unusable, remove.”

- Rerun the metadata and following code and use the “Check the fluxes for which I changed the start time due to lags or ebullition” code chunk to check the fluxes from the Plots you changed.

- Finish running the CO2 code to output processed CO2 data in the Processed_Data folder.

- Open the CH4 code and change the date. Run the code to do the same checks.

- To change the metadata, go back to the CO2 code and make changes in the “Make changes to metadata” chunk for Time_7810 and rerun the code to get new metadata.

- Rerun the CH4 code to finish processing CH4 data and output processed CH4 data in the Processed_Data folder.

- Open the N2O code and change the date. Run the code to do the same checks.

- If necessary, to change the metadata, go back to the CO2 code and make changes in the “Make changes to metadata” chunk for Time_7820 and rerun the code to get new metadata. However, the N2O fluxes will generally be messy and low, so will hopefully not need too much editing.

- Rerun the N2O code to finish processing N2O data and output processed N2O data in the Processed_Data folder.

- Check all processed data to insure it has Plot names and calculated fluxes.

Useful information:

- TEMPEST Autochamber Notes: https://docs.google.com/document/d/16xWvo4vpHII4AkZQdvndoG6ney1uPABS3IqzMNkQD0w/edit?tab=t.0

- TEMPEST Autochamber Checks from the 2026 Event: https://docs.google.com/spreadsheets/d/1PMAp7ZF32q9yo3RSoJHUVwQWiBOk6tY9fVe6QC4b2gw/edit?gid=0#gid=0

- Autochamber Data on Google Drive: https://drive.google.com/drive/folders/1ETfZoritHHmPS0X4vXFLmjBlqMCS8OK8

- STEEP-CF-autochambers GitHub: https://github.com/COMPASS-DOE/STEEP-CF-autochambers/tree/main

- Issue from working through 2026 autochamber flux data: https://github.com/COMPASS-DOE/STEEP-CF-autochambers/issues/4